ALiEditor User Manual

1.3

2011-07-10

THIS DOCUMENT CONTAINS PROPRIETARY TECHNICAL INFORMATION WHICH IS THE PROPERTY OF ALI CORPORATION AND SHALL NOT BE DISCLOSED TO OTHERS IN WHOLE OR IN PART, REPRODUCED, COPIED, OR USED AS THE BASIS FOR DESIGN, MANUFACTURING, OR SALE OF APPARATUS WITHOUT WRITTEN PERMISSION OF ALI CORPORATION

Change History

| Revision | Issue Date | Comments |
| --- | --- | --- |
| 1.0 | 2006-12-7 | First Release for ALiEditor v1.0.2 |
| 1.1 | 2007-3-19 | AliEditor v1.0.7_TF,add upload .udf/.ddf |
| 1.2 | 2009-4-10 | AliEditor DB4 2.1.0a 2008-11-22, support M3602 module |
| 1.3 | 2011-7-10 | AliEditor v3.0.1 2011-07-12, support M3606 combo ui |
| 1.4 | 2011-7-20 | AliEditor v3.0.2 2011-07-20, modify script.ini user guide |

Contents

1	Introduction	1

1.1	Summary	1

1.2	Files	1

2	Operation	2

2.1	Open	2

2.2	Browse	3

2.3	Edit	3

2.3.1	Satellite	3

2.3.2	TP	4

2.3.3	Program	4

2.3.4	Favorite Group	5

2.4	Save	6

2.5	Export	6

2.6	Download File	6

2.7	Download	7

2.8	Upload	7

# Introduction

## Summary

ALi Editor is designed for updating the Default Database of STB Flash. It can also update the User Database because user is very troubled to use remote to edit the User Database.

The ALi Editor data source can be from *.abs/*.ddf file or *.sdx/*.txt. User can create the data source by Manually Edit the ALi Editor or Auto Scan using STB device to get the User Database, also can download the Satcodx data from the web. User can expediently select the Default Database or User Database to download to STB.

Pic. (I-1)

## Files

.abs：The data file created by IDE Tool. Or the data file uploaded from STB Flash to PC.

.ddf：The data file only include Default Database. It can be merged with other data using certain tool.

.udf：The data file only include User Database. It can be merged with other data using certain tool.

.sdx：The file with Satcodx data format.

.txt：The file transferred by a excel with certainly format.

Below is the excel format:

* --- add “*” before a satellite

; --- add “;” to remark one line

Save the excel file with “Text File（Table compart）” to a txt file.

# Operation

## Open

Select the Menu or Tool bar to click the Open item:

Load Template: Load Template file (.h) (dvr_node_m33.h dvr_node_3602.h) to create script.ini.

Open Script: Open script.ini of m33 or m36 chip and choose the right Chip Type.

Open file: There are 6 type files can be selected: *.abs/*.ddf/*.udf/*.ddd/*.sdx/*.txt

When open the *.abs, follow dialog box will appear:

User should select the Default Database or User Database to open.

## Browse

The “Left window” list the Satellite/TP tree/Favorite Group. When select one TP, the “Right window” will list the Programs for this TP.

The “Right top window” is the button to filter TV or Radio.

## Edit

### Satellite

- Add Satellite

- Edit Satellite

Note:

1.In Combo UI project,if the combo is “X + DVB-S”(“ISDBT + DVB-s” for example), we should set the X satellite to the first place as follows:

2.Do not support adding and editing TP&Channel to the X satellite right now.

### TP

- Add TP

- Edit TP

### Program

- Add Program

- Edit Program

Note: Should first select one TP, then user can Insert Channel.

When Insert Channel/Edit Channel, user can make one program to one or more Favorite Groups

Program Information for M33*:

Program Information for M3602:

### Favorite Group

Select one Favorite Group, and then right click the program.

- Rename Group

Modify the Group Name.

- Remove Item

Remove the selected programs from this group.

- Edit Group

The all program are listed in the Favorite Group Edit Menu.

User can remove the Program from this Group or add one program to the Group.

## Save

All type file except *.txt/*.sdx can be saved.

When save *.abs, user need to select saving to default database or user database.

## Export

If want to save the *.txt/*.sdx to database, user should use the export function.

In fact, “Save” is the special case of “Export”.

## Download File

Select the *.abs file, then Download the Database to STB Flash.

Before Download, should select the aim Database which is “Default Database” or “User Database”.

## Download

This is the special case of “Download File”. It just Download the current file to STB Flash. It can’t support to select one file on PC to Download.

## Upload

Upload the data from STB Flash to PC:

When click the “Upload (STB –＞PC)”, a window opened to user to select which file want to be saved.

If the project is compiled with a new bootloader.abs (support above transfer protocol 2.1 ), the save type can be *.udf/*.ddf/*.abs, otherwise the save type can be only *.abs